# Troubleshooting: ASBR Not Advertising a Default Route in OSPF

When configuring an Autonomous System  Boundary Router (ASBR) to inject a default route into OSPF, you typically use:

Code

router ospf <process-id>

default-information originate

However, there are cases where the ASBR fails to advertise the default route, even though the command is present. This often results in routers within the OSPF area dropping packets destined for networks not found in their routing table, since they never receive a 0.0.0.0/0 route.

A common cause is a misconfigured or missing static default route on the ASBR. OSPF will not originate a default route unless one exists (unless you use the always keyword) where the option is possible.

Below is a reliable troubleshooting workflow that has consistently resolved this issue in lab and production environments.

## Why This Happens

OSPF only advertises a default route when:

- A valid static default route exists on the ASBR, and

- default-information originate is configured.

- And the default route was first configured before the default route advertising command is used.

If the static route is missing, incorrect, or was added after OSPF initialization, the ASBR may fail to inject the default route.

# Troubleshooting Steps

## Step 1: Remove the Existing Default Route

Start by removing the current static default route. This clears any stale or incorrect configuration.

Code

R1(config)# no ip route 0.0.0.0 0.0.0.0 <next-hop>

Example:

Code

R1(config)# no ip route 0.0.0.0 0.0.0.0 203.0.113.2

To verify it was removed:

Code

R1# show ip route static

You should no longer see a S* 0.0.0.0/0 entry.

## Step 2: Reconfigure the Default Route

Re-add the correct static default route:

Code

R1(config)# ip route 0.0.0.0 0.0.0.0 <next-hop>

Example:

Code

R1(config)# ip route 0.0.0.0 0.0.0.0 203.0.113.2

## Step 3: Reapply OSPF Default Route Origination

Now re-enable default route injection:

Code

R1(config-router)# default-information originate

If you want the ASBR to advertise a default route even if the static route is missing, use:

Code

R1(config-router)# default-information originate always

## Step 4: Verify OSPF Is Advertising the Default Route

On the ASBR:

Code

R1# show ip ospf database external

You should see an entry for 0.0.0.0.

On another router in the OSPF domain:

Code

R2# show ip route ospf

Look for:

Code

O*E2 0.0.0.0/0 [110/1] via <next-hop>

# Conclusion

When an ASBR fails to advertise a default route, the issue is often tied to the static default route itself. Removing and reconfiguring the route forces OSPF to re-evaluate and correctly originate the default route. This simple reset has proven to be one of the most effective fixes in real-world troubleshooting.

I hope this information has helped you to resolve similar problem you are facing.